| LEICA STELLARIS 5 LIA START-UP PROCEDURE |  |  |
| --- | --- | --- |
| - Switch on the main power (1) - Switch on the laser power (2) - Turn the laser key (3) |  |
| 4) Check that the LED unit is on |  |
| 5) Switch on the software computer, and login with the Windows account BICuser (pw=BICuser). |  |
| 7) Open the internet browser, login into iLab and go to the Kiosk. Start your session. Wait until the status will become . |  |
| 8) Start the Leica X software |  |  |
| 9) For the best imaging results, clean objectives you intend to use before imaging. |  |  |

| OBJECTIVE |  | NA |  | WD |  | Immersion |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5x |  | 0.15 |  | 13.7mm |  | dry |  |
| 10x |  | 0.40 |  | 2.56mm |  | dry |  |
| 20x |  | 0.75 |  | 0.62 mm |  | dry |  |
| 20x Gly |  | 0.75 |  | 0.66 mm |  | Gly |  |
| 40x Gly Corr |  | 1.25 |  | 0.35 mm |  | Gly |  |
| 60x Oil |  | 1.4 |  | 0.140 mm |  | Oil |  |
|  |  |  |  |  | D=2000*NA/mag |  |

| LEICA STELLARIS 5 LIA SHUTDOWN PROCEDURE |  |
| --- | --- |
| Always shut down the instrument at the end of your session unless the next user is standing behind you. |  |
| 1) If an oil or glycerol objective was used, clean the objective with lens paper. Also clean the slide holder on the stage if this has been used. |  |
| 2) Switch to 5x objective. Lower the stage all the way down until the lower Z-limit reached. |  |
| 3) Close the LASX software and finish your session in iLab Kiosk . |  |
| 4) Sign out of BICuser account. Shut down Windows. |  |
| 5) Turn the laser key (3) to off position, followed by switching of the laser unit (2). |  |
| 6) Turn off the main power (1). |  |
| 7) Put on the protective cover to keep the dust away. |  |
| 8) Dispose of used lens paper and other items in the trash boxes situated at the entrance to the facility. |  |