| Zeiss LSM800 CLSM START-UP PROCEDURE |  |  |  |
| --- | --- | --- | --- |
| - Switch on the System switch (1), followed by the Components switch (2). The laser key is ALWAYS on. - Turn on CoolLED unit (3). |  |  |
| DO NOT TOUCH the other switches, they should be on all the time. OBS! It is normal that Definite focus controller fc12 shows OFF on the display. |  |  |
| 3) Switch on computer, and login with the Windows account BICuser (pw=BICuser). |  |  |  |
| 4) Open the internet browser, login into iLab and go to the Kiosk. Start your session. Wait until the status will become . Note that late arrival (+30min) is automatically considered as NO SHOW in iLab, so a new session needs to be created. |  |  |  |
| 5) Start the ZEN blue software . Say yes to stage calibration if asked. |  |  |  |
| 6) For the best imaging results, clean objectives you intend to use. |  |  |  |

| OBJECTIVE | NA | WD | Immersion | Coverglass thickness |
| --- | --- | --- | --- | --- |
| 5x | 0.25 | 12.5mm | dry | 0.17mm |
| 10x | 0.45 | 2.0mm | dry | 0.17mm |
| 20x | 0.8 | 0.55 mm | dry | 0.17mm |
| 40x W | 1.2 | 0.28mm | water | 0.14-0.19mm |
| 40x Oil | 1.2 | 0.21mm | oil | 0.17mm |
| 63x Oil | 1.4 | 0.19mm | oil | 0.17mm |

| Zeiss LSM800 CLSM SHUTDOWN PROCEDURE |  |
| --- | --- |
| Always shut down the instrument at the end of your session unless the next user is standing behind you. |  |
| 1) If an oil or water objective is used, clean the objective with lens paper. Also clean the slide holder on the stage if this has been used. |  |
| 2) Switch to 5x objective. Lower the stage until the lower Z-limit reached. |  |
| 3) Close the ZEN software and finish your session in iLab Kiosk . |  |
| 4) Sign out of BICuser account. Shut down Windows. |  |
| 5) Switch off the CoolLED lamp unit. |  |
| 6) Switch off the Components, followed by the System switch. The laser key remains on! |  |
| 7) Put on the dust protective cover. |  |
| 8) Dispose of used lens paper and other items in the trash boxes situated at the entrance to the facility. |  |