| Olympus IX73 START-UP PROCEDURE |  |  |
| --- | --- | --- |
| 1) On the right side, turn on Olympus light power supply (1) and control box (2). Check that the Tango stage controller is on. |  |  |
| 2) Switch on the software computer, and login with the Windows account BICuser (pw=BICuser). |  |  |
| 3) Open the internet browser, login into iLab and go to the Kiosk. Start your session. Wait until the status will become |  |  |
| 4) Start the cellSens Dimention software |  |  |
| 5) To view fluorescence, on the left side turn on the pE-300 LED unit (3) (ON appears at the left upper corner on the display). To view transmitted light, turn on the light source on the microscope stand (4). |  |  |
| 6) For the best imaging results, clean objectives you intend to use before imaging. |  |  |

| OBJECTIVE | NA | Manual magnif* | WD (mm) | Immersion |
| --- | --- | --- | --- | --- |
| 2x | 0.08 | 2X | 6.2 | dry |
| 4x | 0.13 | 1.6X | 17 | dry |
| 10x | 0.30 | 1.6X | 9.5 | dry |
| 20x | 0.45 | 1X | 6.6-7.8 | dry |
| 20x | 0.8 | 1X | 0.6 | dry |
| 40x | 0.6 | 1X | 2.7-4.0 | dry |

*Recommended.

| Olympus IX73 SHUTDOWN PROCEDURE |  |
| --- | --- |
| Always shut down the instrument at the end of your session unless the next user is standing behind you. |  |
| 1) Switch to 2x objective. Lower the stage. |  |
| 2) Close the cellSens software and finish your session in iLab Kiosk . |  |
| 3) Sign out of BICuser account. Shut down Windows. |  |
| 5) Switch off the brightfield lamp (4) and pE-300 LED unit (3). |  |
| 6) Switch off the Olympus control box (2) and light poser supply (1). OBS!!! Do not switch off the Tango stage controller. |  |
| 7) Put on the dust protective cover. |  |
| 8) Dispose of used lens paper and other items in the trash boxes situated at the entrance to the facility. |  |