| Molecular Devices ImageXpress HCS.ai START-UP PROCEDURE |  |
| --- | --- |
| - Switch on the main power (1) (at the left side of the instrument) - If the water objectives will be used, check the water level in the bottles with yellow lids (“Waste” and “Supply”). - Press the button to load the samples (can also be done later) OBS! Environmental controls (temperature, CO2) can be started only from the software. |  |
| 4) Switch on the software computer, and login with the Windows account BICuser (pw=BICuser). |  |
| 7) Open the internet browser, login into iLab and go to the Kiosk. Start your session. Wait until the status will become . |  |
| 8) First start the InCarta analysis software, and then MetaXpress Acquire. |  |

| OBJECTIVE |  | NA |  | WD | Immersion |  |
| --- | --- | --- | --- | --- | --- | --- |
| 4x |  | 0.2 |  | 20 mm | dry |  |
| 10x |  | 0.45 |  | 4 mm | dry |  |
| 20x Corr |  | 0.45 |  | 8.2-6.9mm | dry |  |
| 20x W Corr |  | 0.95 |  | 0.99-0.9 mm | water |  |
| 40x Corr |  | 0.8 |  | 3.6-2.8 mm | dry |  |
| 40x W Corr |  | 1.15 |  | 0.61-0.59 mm | water |  |
|  |  |  |  |  | D=2000*NA/mag |

| Molecular Devices ImageXpress HCS.ai SHUT-DOWN PROCEDURE |  |
| --- | --- |
| Always shut down the instrument at the end of your session unless the next user is standing behind you. |  |
| 1) Take away your samples. |  |
| 2) Turn off incubation if that was used |  |
| 2) Close the MetaXpress and InCarta software. |  |
| 2) Finish your session in iLab Kiosk. |  |
| 4) Sign out of BICuser account. Shut down Windows. |  |
| 6) Turn off the power (1). |  |
| 7) Check that the place is tidy and dispose any trash to the trash boxes situated at the entrance to the facility. |  |